/*! \page index GEL for 68HC11 and 68HC12

GEL is a collection of embedded libraries and examples for

68HC11 and 68HC12.  Several libraries are general purpose

libraries that are or may be ported on various boards.  Some

libraries are more specific due to the hardware they control.

GEL also contains several examples indended to show how to use

the GNU development chain for 68HC11 and 68HC12.

GEL is based arround a small framework intended to help to use

the libraries, its examples, build them, port them to new boards

and provide documentation.

They come with a framework to build them.  This framework

consists of:

<ul>

<li>A set of includes that try to hide the details of

the board implementation and the details of the micro-controller.

<li>A utility library which contains useful functions for

the examples (printing on the serial line for example),

<li>A simple make configuration environment to automate

and help in building the examples.

<li>A documentation framework based on \b doxygen.

</ul>

Examples and the library are presented more into details

in the following sections:

<ul>

<li>Section \ref build

<li>Section \ref porting

<li>Section \ref utilLibrary

<li>Section \ref bootExamples

<li>Section \ref smallExamples

<li>Section \ref largeExamples

<li>Section \ref benchExamples

</ul>

The following libraries are available:

<ul>

<li>\ref libc

<li>\ref libbsp

<li>\ref libutil

<li>\ref libgel

<li>\ref libbot

</ul>

GEL can be downloaded from the following site:

<A HREF="http://gel.sourceforge.net/download.php">

http://gel.sourceforge.net/download.php

</A>

*/

/*! \page build Build the examples

The examples are ported on several boards. You can select the board

on the command line with \b make or by changing the \em board.def

file.

The following boards are supported:

<TABLE BORDER="1" WIDTH="95%">

<TR>

<TD WIDTH="30%" VALIGN="TOP">m68hc11-32k</TD>

<TD>68HC11 generic board with 32K ram and 32K rom

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x1040..0x7fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x7fff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x8000..0xffff</TD>

</TR>

</TABLE>

<p>

Prefer this board if you want to use the simulator.

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc11-cme11</TD>

<TD>Axiom Manufacturing CME11 board with Buffalo monitor

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x1040..0x1fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x1fff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x2000..0x7fff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">bootstrap</TD>

<TD>68HC11 bootstrap mode

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x0ff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x00..0x0ff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">eeprom-512</TD>

<TD>68HC11 512 eeprom mode

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x0ff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0xB600..0xB7FF</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc11-ebcs</TD>

<TD>68HC11 board with 8K ram (SCz board)

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0xE000..0xE1ff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x0ff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0xE200..0xFfff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc11-6270</TD>

<TD>MIT 6.270 68HC11 board

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0xE000..0xE1ff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0xFFBF</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x8000..0xDfff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc12-axcmd12</TD>

<TD>Axiom Manufacturing CMD12 board with DBug monitor

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x2000..0x2fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x3000</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x3000..0x8000</TD>

</TR>

</TABLE>

</TD>

</TR>

</TABLE>

The board is configured either by specifying it to the command line

or by updating the file config/board.def. For the first method,

type the following command:

<pre>

make TARGET_BOARD=board-name

</pre>

where \em board-name is the name of the board picked from the

table above.

Otherwise, in config/board.def change the line:

<pre>

TARGET_BOARD=

</pre>

to put the board you selected.

Before launching \b make you should make sure the GNU Binutils

and Gcc executables are in your path. Then, proceed with the following

commands:

<pre>

make

</pre>

or

<pre>

make TARGET_BOARD=<i>board-name</i>

</pre>

This will build the examples in all the sub-directories.

To execute the program, .... you have some choice:

<TABLE BORDER="0">

<TR><TD WIDTH="30%" VALIGN="TOP">

Simulator

</TD><TD>	m6811-elf-run <prog>.elf

<br>	      or

<br>		m6811-elf-run -v <prog>.elf

</TD></TR>

<TR><TD VALIGN="TOP">

Gdb Sim

</TD><TD>

<pre>	m6811-elf-gdb <prog>.elf

(gdb) target sim

(gdb) load <prog>.elf

(gdb) run

</pre>

</TD></TR>

<TR><TD VALIGN="TOP">

Gdb Buffalo

</TD><TD><pre>  	m6811-elf-gdb <prog>.elf

(CME11)	(gdb) set remotebaud 9600

(gdb) target buffalo /dev/ttyS2

(gdb) load <prog>.elf

(gdb) run

</pre>

</TD></TR>

<TR><TD VALIGN="TOP">

Gdb Dbug

</TD><TD><pre>	m6811-elf-gdb <prog>.elf

(CMD12)	(gdb) set remotebaud 9600

(gdb) target dbug /dev/ttyS2

(gdb) load <prog>.elf

(gdb) run

</pre>

</TD></TR>

<TR><TD VALIGN="TOP">

Bootstrap

</TD><TD>	Upload the program (either <prog>.elf, or <prog>.s19)

with your favorite target manager.

</TD></TR>

<TR><TD VALIGN="TOP">

Prom

</TD><TD>	Burn the <prog>.b in some prom.

</TD></TR>

</TABLE>

*/

/*! \page utilLibrary Utility Library

To increase the portability of programs among several boards,

the utility library provides a set of includes and functions

that hide these differences.  It also provides a set of commonly

used functions such as benchmarking, and other general purpose

operations.

<p>

To use this library, you must:

<ul>

<li>Add a <b>-I<i>install-dir</i>/include</b> directive to gcc and cpp,

<li>Link with <b><i>install-dir</i>/lib/libutil.a</b>.

</ul>

The utility library provides the following modules:

<ul>

<li>\ref locks

<li>\ref sio

<li>\ref interrupts

<li>\ref imath

<li>\ref benchs

</ul>

*/

/*! \page bootExamples Bootstrap Examples

These examples are very small and they are designed to fit in less

than 256 bytes (less than 240 to leave room for the stack).  They

represent typical programs that can be uploaded during the 68HC11

bootstrap mode.

<ul>

<li>\ref hello

<li>\ref blink

<li>\ref boot_hello

<li>\ref booter

<li>\ref checker

<li>\ref dumper

<li>\ref burner

</ul>

In general to compile and link these programs, you will use the

following %command:

<pre>

m6811-elf-gcc -o file.elf -nostdlib -nostartfiles \

-Wl,-defsym,_io_ports=0x1000 \

-Wl,-defsym,_.tmp=0x0 -Wl,-defsym,_.z=0x2 \

-Wl,-m,m68hc11elfb \

-mshort -Os -fomit-frame-pointer -msoft-reg-count=0 \

file.c

</pre>

In this cryptic and long %command, the following options are

used by the compiler:

<TABLE>

<TR>

<TD>

-mshort

</TD>

<TD>

This option tells gcc to use 16-bit integers.

</TD>

</TR>

<TR>

<TD>

-Os

</TD>

<TD>

This option tells gcc to optimize for size.

</TD>

</TR>

<TR>

<TD>

-fomit-frame-pointer

</TD>

<TD>

This option tells gcc to remove the frame pointer.

</TD>

</TR>

<TR>

<TD>

-msoft-reg-count=0

</TD>

<TD>

This option tells gcc to avoid to use soft registers

(except _.tmp and _.z which are sometimes necessary)

</TD>

</TR>

</TABLE>

And the following options are used by the linker:

<TABLE>

<TR>

<TD>

-nostdlib

</TD>

<TD>

The standard libraries are not passed to the linker.

</TD>

</TR>

<TR>

<TD>

-nostartfiles

</TD>

<TD>

The standard startup files are not used during the link.

</TD>

</TR>

<TR>

<TD>

-defsym _io_ports=0x1000

</TD>

<TD>

The linker will define the symbol \c _io_ports and

assign it the value \c 0x1000.  This symbol is used

by the examples to access the 68HC11 registers.

</TD>

</TR>

<TR>

<TD>

-defsym _.tmp=0x0<br>

-defsym _.z=0x0

</TD>

<TD>

This will define the symbol \c _.tmp used by gcc.

</TD>

</TR>

</TABLE>

*/

/*! \page smallExamples Small Examples

<ul>

<li>\ref simple

<li>\ref trap

<li>\ref timerExample

<li>\ref stack_depth

<li>\ref analyzer

<li>\ref freqmeter

<li>\ref pulse

</ul>

*/

/*! \page largeExamples Larger Examples

<ul>

<li>\ref calculator

<li>\ref benchExamples

<li>\ref persist

<li>\ref interpretor

</ul>

*/

/*! \page benchExamples Benchmark Examples

The benchmark programs are intended to measure the efficiency of

the generated code.

<ul>

<li>\ref bench-basic

<li>\ref bench-string

<li>\ref bench-float

<li>\ref bench-misc

</ul>

*/